PROJECT 2

Anti-Sleep Buzzer Alarm with OpenCV & Bluetooth

Eye Detection System using Python + HC-05 + Buzzer

# 1. Experiment

This project creates an anti-sleep alarm system for people studying or working long hours in front of a computer. The system continuously monitors the user's eyes through the laptop webcam. When the user falls asleep (eyes closed for 2+ seconds), a buzzer connected to Arduino beeps loudly to wake them up.

The system uses OpenCV and MediaPipe library to detect the user's face and eyes in real time. The Eye Aspect Ratio (EAR) algorithm measures how open or closed the eyes are. When EAR drops below the threshold for 2 seconds, Python sends an ALARM command via Bluetooth to the Arduino, which triggers the buzzer. When the user opens their eyes, the buzzer stops automatically.

# 2. Requirements

## Software Requirements

- Python 3.11 or above

- Arduino IDE 2.3 or above

- VS Code (recommended editor)

- Python Libraries: opencv-python, mediapipe, pyserial

- Arduino Library: SoftwareSerial (built-in)

## Hardware Requirements

- Arduino Uno board

- HC-05 Bluetooth module

- Active Buzzer (5V)

- Laptop with webcam

- USB cable for programming

- Jumper wires

# 3. Components

Component

Arduino Uno

HC-05 Bluetooth

Active Buzzer

Jumper Wires

USB Cable

Laptop for camera

On/off switch

18650 battery -2

# 4. Connectivity / Wiring

## HC-05 Bluetooth Module to Arduino

| HC-05 Pin | Arduino Pin | Notes |
| --- | --- | --- |
| VCC | 5V | Power supply |
| GND | GND | Ground |
| TX | Pin 10 (RX) | HC-05 sends to Arduino |
| RX | Pin 11 (TX) | Arduino sends to HC-05 |

## Buzzer to Arduino

| Buzzer Pin | Arduino Pin | Notes |
| --- | --- | --- |
| + (longer leg) | Pin 8 | Signal from Arduino |
| - (shorter leg) | GND | Ground connection |

## Important Wiring Notes

- Active buzzer only - has small circuit board on back

- Buzzer longer leg (+) MUST go to Pin 8, shorter leg (-) to GND

- Wrong polarity = no sound even if code is correct

- HC-05 TX and RX must be disconnected before uploading Arduino code

# 5. Steps Procedure

## Step 1 - Install Python Libraries

pip install mediapipe==0.10.9

pip install opencv-python pyserial

## Step 2 - Pair HC-05 with Laptop Bluetooth

- Power on Arduino with HC-05 connected (LED blinks fast)

- Windows 11: Settings > Bluetooth > Add Device > HC-05

- Enter PIN: 1234(connect fast)

- If pairing fails: use Control Panel > Devices & Printers > Add Device

- Open Device Manager > Ports > Note the COM port number (e.g. COM7)

## Step 3 - Upload Arduino Code

- Disconnect HC-05 TX and RX pins (Pin 10 and Pin 11) from Arduino

- Open Arduino IDE, select Board: Arduino UNO and Port: COM3

- Upload the Arduino code provided below

- After upload complete, reconnect HC-05 TX to Pin 10 and RX to Pin 11

## Step 4 - Update Python Code

- Open python file in VS Code

- Change BLUETOOTH_PORT = 'COM7' to your actual COM port number

- Save the file

## Step 5 - Run the Project

- Power on Arduino

- Sit in front of laptop camera

- Run Python: python alarm_control.py

- Eyes open = complete silence, system monitoring

- Eyes closed 2+ seconds = Buzzer BEEPS loudly

- Eyes open again = Buzzer STOPS automatically

# 6. Code

## Arduino Code (EYE_ANTISLEEP_ALARM.ino)

#include <SoftwareSerial.h>

SoftwareSerial BT(10, 11); // RX, TX

int BUZZER = 8;

void setup() {

pinMode(BUZZER, OUTPUT);

digitalWrite(BUZZER, LOW);

BT.begin(9600);

Serial.begin(9600);

Serial.println('Arduino Ready!');

}

void loop() {

if (BT.available()) {

char cmd = BT.read();

Serial.println(cmd);

if (cmd == 'A') {

tone(8, 1000);

Serial.println('BUZZER ON');

} else if (cmd == 'S') {

noTone(8);

Serial.println('BUZZER OFF');

}

}

}

## Python Code (alarm_control.py)

import cv2

import mediapipe as mp

import serial

import time

BLUETOOTH_PORT = 'COM7'

BAUD_RATE      = 9600

EAR_THRESHOLD  = 0.25

CLOSED_SECONDS = 2.0

mp_face_mesh = mp.solutions.face_mesh

face_mesh = mp_face_mesh.FaceMesh(

max_num_faces=1, refine_landmarks=True,

min_detection_confidence=0.5,

min_tracking_confidence=0.5

)

LEFT_EYE  = [362, 385, 387, 263, 373, 380]

RIGHT_EYE = [33, 160, 158, 133, 153, 144]

def eye_aspect_ratio(landmarks, eye_points, w, h):

points = []

for i in eye_points:

x = int(landmarks[i].x * w)

y = int(landmarks[i].y * h)

points.append((x, y))

A = ((points[1][0]-points[5][0])**2+(points[1][1]-points[5][1])**2)**0.5

B = ((points[2][0]-points[4][0])**2+(points[2][1]-points[4][1])**2)**0.5

C = ((points[0][0]-points[3][0])**2+(points[0][1]-points[3][1])**2)**0.5

return (A + B) / (2.0 * C)

bt = serial.Serial(BLUETOOTH_PORT, BAUD_RATE, timeout=1)

time.sleep(2)

cap = cv2.VideoCapture(0)

alarm_on = False

eyes_closed_start = None

print('Anti-Sleep Alarm Started!')

while True:

ret, frame = cap.read()

if not ret: break

h, w = frame.shape[:2]

rgb = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)

results = face_mesh.process(rgb)

eyes_open = True

if results.multi_face_landmarks:

landmarks = results.multi_face_landmarks[0].landmark

left_ear  = eye_aspect_ratio(landmarks, LEFT_EYE, w, h)

right_ear = eye_aspect_ratio(landmarks, RIGHT_EYE, w, h)

ear = (left_ear + right_ear) / 2.0

if ear < EAR_THRESHOLD:

eyes_open = False

if eyes_closed_start is None:

eyes_closed_start = time.time()

closed_duration = time.time() - eyes_closed_start

if closed_duration >= CLOSED_SECONDS and not alarm_on:

bt.write(b'A')

alarm_on = True

print('ALARM ON - Wake up!')

else:

eyes_closed_start = None

if alarm_on:

bt.write(b'S')

alarm_on = False

print('Eyes open - ALARM OFF')

cv2.imshow('Anti-Sleep Alarm', frame)

if cv2.waitKey(1) & 0xFF == ord('q'): break

bt.write(b'S')

cap.release()

cv2.destroyAllWindows()

bt.close()

# 7. Where to Buy (India)

| Component | Website | Price |
| --- | --- | --- |
| Arduino Uno | https://robocraze.com/products/uno-smd-board-compatible-with-arduino | 229 |
| HC-05 Bluetooth | https://robocraze.com/products/hc-05-bluetooth-module | 224 |
| Active Buzzer 5V | https://robu.in/product/5v-passive-buzzer-5pcs/#tab-robu_other_tab | 38 |
| Jumper Wires kit | https://robocraze.com/products/jumper-wire-set-m2m-m2f-f2f-40-pcs-each | 128 |
| USB cable | https://robocraze.com/products/usb-a-b-cable-3-0-cm | 33 |
| 18650 batteries - 2 | https://robocraze.com/products/3-7v-1200mah-18650-battery | 40 |
| 18650 battery case | https://robocraze.com/products/18650-2-cell-holder | 21 |
| On/off switch | https://robocraze.com/products/2-pin-mini-on-off-spst-rocker-switch-19-x-13mm | 25 |
|  | Total | 738˜800 |

Circuit diagram